class Command(Enum):

MOTOR_AZ_STOP = 1

MOTOR_SWEEP = 2

MOTOR_AZ_RELEASE = 3

MOTOR_AZ_GOTO_ENCODER_POS = 4

MOTOR_AZ_GOTO_ABSOLUTE_POS = 5

MOTOR_AZ_GOTO_ANGLE_POS = 6

MOTOR_AZ_MOVE_STEP = 7

MOTOR_AZ_MOVE_VELOCITY_MODE = 8

MOTOR_EL_STOP = 9

MOTOR_EL_GOTO_ABSOLUTE_POS = 11

MOTOR_EL_GOTO_ANGLE_POS = 12

MOTOR_EL_MOVE_STEP = 13

MOTOR_EL_MOVE_VELOCITY_MODE = 14

class Udat():

def __init__(self, log, com_port):

self.mbus_dev = Modbus(log, com_port)

def get_firmware(self):

return self.mbus_dev.read_holding_register_u64(40001)

def get_nanotec_firmware(self):

nanotec_firmware = ""

for i in range(17):

fir_digit = self.mbus_dev.read_holding_register_u16(40019 + i)

nanotec_firmware += chr(fir_digit)

return nanotec_firmware

def get_alive_counter(self):

return self.mbus_dev.read_holding_register_s32(40005)

def get_encoder_value(self):

return self.mbus_dev.read_holding_register_s32(40017)

def get_actual_az_angle(self):

return self.mbus_dev.read_holding_register_s32(40065) / 1000

def get_actual_el_angle(self):

return self.mbus_dev.read_holding_register_s32(40067) / 1000

def get_gps_data(self):

year, mon, day, hour, min, sec, dummy1, dummy2 = self.mbus_dev.read_holding_registers_u128(40009)

nsec = self.mbus_dev.read_holding_register_u64(40015)

# year = self.mbus_dev.read_holding_register_u16(40009)

# mon = self.mbus_dev.read_holding_register_u16(40010)

# day = self.mbus_dev.read_holding_register_u16(40011)

# hour = self.mbus_dev.read_holding_register_u16(40012)

# min = self.mbus_dev.read_holding_register_u16(40013)

# sec = self.mbus_dev.read_holding_register_u16(40014)

return year, mon, day, hour, min, sec, nsec

def get_system_status(self):

return self.mbus_dev.read_holding_register_u64(40036)

def status_details(self):

status = self.get_system_status()

Utility.show_val("azcw_limit", Utility.get_bit(status, 1))

Utility.show_val("azccw_limit", Utility.get_bit(status, 2))

Utility.show_val("el_up_limit", Utility.get_bit(status, 3))

Utility.show_val("el_down_limit", Utility.get_bit(status, 4))

# Utility.show_val("az_motor_ok", Utility.get_bit(status, 5))

# Utility.show_val("el_motor_ok", Utility.get_bit(status, 6))

# Utility.show_val("pol_motor_ok", Utility.get_bit(status, 7))

def goto_az_encoder(self, target_encoder):

self.mbus_dev.write_holding_registers(40040, target_encoder, DataType.INT32T)

return self.apply_command(Command.MOTOR_AZ_GOTO_ENCODER_POS)

def move_step(self, step_deg):

self.mbus_dev.write_holding_registers(40044, Utility.x1000_ang_to_count(step_deg), DataType.INT32T)

return self.apply_command(Command.MOTOR_AZ_MOVE_STEP)

def apply_command(self, command):

return self.mbus_dev.write_holding_registers(40064, command.value, DataType.UINT16T)

def right_angle(self, angle):

return self.mbus_dev.write_holding_registers(40056, Utility.x1000_ang_to_count(angle), DataType.INT32T)

def left_angle(self, angle):

return self.mbus_dev.write_holding_registers(40058, Utility.x1000_ang_to_count(angle), DataType.INT32T)

def angle_calibration(self, az_count, el_count, az_angle, el_angle):

self.mbus_dev.write_holding_registers(40069, az_count, DataType.INT32T)

self.mbus_dev.write_holding_registers(40071, el_count, DataType.INT32T)

self.mbus_dev.write_holding_registers(40073, Utility.x1000_ang_to_count(az_angle), DataType.INT32T)

self.mbus_dev.write_holding_registers(40075, Utility.x1000_ang_to_count(el_angle), DataType.INT32T)

def software_limits(self, az_cw, az_ccw, el_up, el_down):

self.mbus_dev.write_holding_registers(40087, Utility.x1000_ang_to_count(az_cw), DataType.INT32T)

self.mbus_dev.write_holding_registers(40089, Utility.x1000_ang_to_count(az_ccw), DataType.INT32T)

self.mbus_dev.write_holding_registers(40091, Utility.x1000_ang_to_count(el_up), DataType.INT32T)

self.mbus_dev.write_holding_registers(40093, Utility.x1000_ang_to_count(el_down), DataType.INT32T)

def velocity_mode_speed(self, speed_as_degree_second):

return self.mbus_dev.write_holding_registers(40083, Utility.x1000_ang_to_count(speed_as_degree_second), DataType.INT32T)

def goto_el_absolute(self, target_pos):

self.mbus_dev.write_holding_registers(40054, target_pos, DataType.INT32T)

return self.apply_command(Command.MOTOR_EL_GOTO_ABSOLUTE_POS)

def set_sweep_speed(self, sweep_speed):

self.mbus_dev.write_holding_registers(40085, Utility.x1000_ang_to_count(sweep_speed), DataType.UINT32T)

def get_resolvers(self):

az = self.mbus_dev.read_holding_register_u32(40095)

el = self.mbus_dev.read_holding_register_u32(40097)

return az, el

def goto_az_count(self, angle):

self.mbus_dev.write_holding_registers(40052, Utility.x1000_ang_to_count(angle), DataType.INT32T)

self.apply_command(Command.MOTOR_AZ_GOTO_ABSOLUTE_POS)

def goto_az_angle(self, angle):

self.mbus_dev.write_holding_registers(40048, Utility.x1000_ang_to_count(angle), DataType.INT32T)

self.apply_command(Command.MOTOR_AZ_GOTO_ANGLE_POS)

def goto_el_count(self, count):

self.mbus_dev.write_holding_registers(40054, count, DataType.INT32T)

self.apply_command(Command.MOTOR_EL_GOTO_ABSOLUTE_POS)

def goto_el_angle(self, angle):

self.mbus_dev.write_holding_registers(40050, Utility.x1000_ang_to_count(angle), DataType.INT32T)

self.apply_command(Command.MOTOR_EL_GOTO_ANGLE_POS)