YNK

Remote Monitor and Control Protocol

Ver 1.9 / 10.10.2022

| VERSION HISTORY Version | Date / Sign | Version Description |
| --- | --- | --- |
| V 1.0 | 26.12.2016 / M.F.Okutan | Initial Release. |
| V 1.1 | 05.01.2017 / M.F.Okutan | Start of message byte was added. |
| V 1.2 | 11.08.2017 / M.F.Okutan | Message structures updated. New commands were added. |
|  | 24.08.2017 / M.F.Okutan | Time stamped position command added. Textual changes. Tracking enable/disable added to status command. Tracking enable/disable command added. |
| V 1.3 | 28.08.2017 / M.F.Okutan | Milisecond region added to TPOS command. |
|  | 27.09.2017 / M.F.Okutan | Daily repeat removed. Milisecond added to “TIME?” command. |
| V 1.4 | 19.10.2017 / M.F.Okutan | Parameter descriptions added. Redundan items removed. Clock update command added. Query GPS Settings command added. Set GPS Settings command added. GPS communicaton error added to errstatus. |
| V 1.5 | 14.12.2017 / M.F.Okutan | TLE file upload procedure is added. |
| V 1.6 | 18.12.2017 / M.F.Okutan | Query (22) and set (23) satellite settings commands were added. Some color and statement corrections were made in set gps settings (21) command. |
| V 1.7 | 18.06.2018 / M.F.Okutan | UTC based operation notes added to relevant commands. |
| V 1.8 | 08.02.2022 / Semih Meşe | Additions and adjustments were made for the transition to X,Y mechanics. |
| V 1.9 | 10.10.2022 / A. Ozdogru | Additions and adjustments were made for the AST Customer Application. |

- Contents

Abbreviations	4

COMMUNICATION PROTOCOL	5

1.	PROTOCOL	5

2.	MESSAGE STRUCTURE	5

3.	DOCUMENT CONVENTIONS	6

COMMUNICATION MESSAGES	6

1.	QUERY VERSION	6

2.	MOVE	6

3.	MOVE STEP	7

4.	GO TO POSITION	7

5.	STOP	8

6.	QUERY STATUS	8

7.	QUERY FAULT/WARNING STATUS	8

8.	QUERY CLOCK INFO	9

9.	SET CLOCK	10

10.	QUERY START/STOP TIME SETTINGS	10

11.	SET START/STOP TIME SETTINGS	11

12.	QUERY GENERAL SETTINGS	11

13.	SET GENERAL SETTINGS	12

14.	QUERY ANGLE SETTINGS	12

15.	SET ANGLE SETTINGS	13

16.	QUERY GPS INFO	13

17.	QUERY TIME-STAMPED POSITION	14

18.	TRACKING START/STOP	14

19.	CLOCK UPDATE BY GPS	15

20.	QUERY GPS SETTINGS	15

21.	SET GPS SETTINGS	15

22.	QUERY SATELLITE SETTINGS	16

23.	SET SATELLITE SETTINGS	16

TLE FILE UPLOAD	18

1.	TLE FILE UPLOAD OPERATION	18

# Abbreviations

SATRAK(YNK)	SVS Satellite Tracker Antenna Controller Unit

ACU 		Antenna Controller Unit

AZ		Azimuth

EL		Elevation

Xor		Exclusive Or

AZCW		Azimuth Clockwise

AZCCW		Azimuth Counter-Clockwise

ELUP		Elevation Up

ELDOWN	Elevation Down

# COMMUNICATION PROTOCOL

## PROTOCOL

Satrak ACU communicates with TCP/IP protocol. Client software initiates a tcp socket connection, then sends query or request commands and receives response command messages from the Satrak ACU. Client software connects to Satrak ACU with IP and tcp port information of the devices. TCP port of the Satrak ACU is 4001 for the remote communication.

## MESSAGE STRUCTURE

$DATETIMEG?=,2016,12,22,15,55,40,10*6F<CR><LF>

Header (1 byte) + Payload + Asterisk (1 byte) + Checksum (2 bytes) + Footer (2 bytes)

All messages are ASCII based. All messages start with $ character, followed by payload and end with asterisk, 2 bytes checksum (1 byte hex), CR (hex. 0x0D), LF (hex. 0x0A) characters respectively.

Message Structure:

$DATETIMEG?=,2016,12,22,15,55,40,10*6F<CR><LF>

Figure 1.1

Message contains header character ($), command, command parameter, checksum and CR-LF characters. Cheksum is calculated executing xor logic through all command and command parameter characters. Asterisk before the checksum and header character is not included in checksum calculation. Note that CR and LF characters are 2 bytes only.

Above example has decimal 111 (hex 6F) checksum byte which is acquired xoring all bytes in command and parameters.

## DOCUMENT CONVENTIONS

In this document, all communication messages are represented italic, query or request messages are blue color, response messages are green color. Some responses in commands chapter of this document should be considered as an example, the response in real target device may be different. Header, checksum, asterisk character and CR/LF characters are not represented with every command explanation in following commands description chapters of this document.

Positive response format without parameter: MOVE=,OK

Negative response format: MOVE=,ERR,ERRCODE

OK: Operation success.

ERR: Operation error.

ERRCODE: Error code

# COMMUNICATION MESSAGES

## QUERY VERSION

Function: This command queries device firmware version.

Query: VERSION?

Response: VERSION?,SATRAK CU V1.2 18/10/2017,SATRAK DU V1.22 18/10/2017

## MOVE

Function: This issues a command to move antenna in X +/-  and Y +/- axes.

Request: MOVE=,X1,X2

X1: X travel speed (Positive: +, Negative: -), deg/s

X2: Y travel speed (Positive: +, Negative: -), deg/s

Example1: Move antenna in Y + with speed of 2 deg/s

Request: MOVE=,0,2

Example2: Move antenna in X - with speed of 3.5 deg/s

Request: MOVE=,-3.5,0

Example3: Move antenna in X + and Y - with speed of 1 deg/s

Request: MOVE=,1,-1

Recommendation Note: In client M&C software, when the move button is pressed, send related MOVE command (i.e. MOVE=,-2,0) and when the move button is released, send related MOVE command (i.e. STOP=) to stop the antenna.

Thus, there is no need to send any command during the button is pressed and hold. It is enough to send command when the button is pressed and released only.

## MOVE STEP

Function: This issues a command to move antenna automatically in X and Y as a certain amount of the given deltaX angle value. This is considered as step by step movement.

Request: STEP=,X1,X2

X1: X step (Positive: +, Negative: -), deg/s

X2: Y step (Positive: +, Negative: -), deg/s

Example1: Move antenna 2 degrees to X +.

Request : STEP=,+2,0

Example2: Move antenna 2 degrees to X -.

Request : STEP=,-2,0

Example3: Move antenna 1 degrees to Y -.

Request: STEP=,0,-1

Example3: Move antenna 1 degree to Y - and X +.

Request: STEP=,1,-1

Recommendation Note: This command can be used as a precise movement in manual move buttons of the M&C software. In every pressing in manual move button, antenna will move by the step defined in the command, not more.

## GO TO POSITION

Function: This issues a command to move antenna automatically to a certain position in azimuth and elevation with angle based.

Request: GOTO=,X1,X2

X1: Target azimuth angle

X2: Target elevation angle

Example1: Position antenna to the azimuth 125.1 and elevation 32.6 automatically.

Request: GOTO=,125.1,32.6

Example2: Position antenna to the azimuth 74.61 automatically and don’t move in elevation.

Request: GOTO=,74.61,NONE

Example3: Position antenna to the elevation 40.5 automatically and don’t move in azimuth.

Request: GOTO=,NONE,40.5

## STOP

Function: This issues a command to stop the antenna movement.

Request: STOP=

## QUERY STATUS

Function: This command reads the status of the Satrak system.

Query: STATUS?

Response: STATUS?,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11,X12,X13,X14,X15

Parameters:

X1: Fault or warning (1: Exist, 0: Non exist)

X2: Az angle

X3: El angle

X4: X + limit (1:Yes, 0:No)

X5: X - limit (1:Yes, 0:No)

X6: Y + limit (1:Yes, 0:No)

X7: Y - limit (1:Yes, 0:No)

X8: Tracking enable/disable status (1:Enabled, 0:Disabled)

X9: Tracking status (0:Not in Track Time, 1:In Track Time, 2:Track Completed, 3:Start Before Time)

X10: Rtc updated with gps status (1:Updated, 0: Not updated)

X11: Gps lock delay (Seconds)

X12: X motion status (-1:X -, 0:No Movement, 1:X +)

X13: Y motion status (-1:Y -, 0:No Movement, 1:Y +)

X14: X angle

X15: Y angle

X16: Auto Track On/Off (1: On, 0: Off)

Example:

Request : STATUS?

Response: STATUS?0,299.49,34.35,0,0,0,0,0,0,0,0,0,0,80.0,35.0,1

Recommendation Note: If the “fault or warning” info is 1, then the “GET FAULT/WARNING STATUS” command can be queried, otherwise not needed.

## QUERY FAULT/WARNING STATUS

Function: This command reads the fault/warning status of the Satrak system.

Query: ERRSTATUS?

Response: ERRSTATUS?,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11,X12

Parameters:

X1: Internal communication error status (1: Error, 0:No error)

X2: TLE file not uptodate (1:Not uptodate, 0: Uptodate)

X3: TLE file not found (1: Not found, 0:Found)

X4: Satellite name not found (1: Not found, 0:Found)

X5: TLE data reading from Web error (1: Error, 0:No error)

X6: Tracking stopped error (1: Error, 0:No error)

X7: Setup error (1: Error, 0:No error)

X8: X motor error (1: Error, 0:No error)

X9: Y motor error (1: Error, 0:No error)

X10: X position sensor error (0:No error, 1:Config Error, 2:Connection Error)

X11: Y position sensor error (0:No error, 1:Config Error, 2:Connection Error)

X12: GPS communication error (1: Error, 0:No error)

Example:

Request : ERRSTATUS?

Response: ERRSTATUS?,0,0,0,0,0,0,0,0,0,0,0

Recommendation Note: If the “fault or warning” info of “GET STATUS” command is 1, then “GET FAULT/WARNING STATUS” command can be queried, otherwise not needed.

## QUERY CLOCK INFO

Function: This command reads the RTC (real-time clock) date and time information of the Satrak ACU.

Query: TIME?

Response: TIME?,X1,X2,X3,X4,X5,X6,X7

Parameters:

X1: Hour

X2: Minute

X3: Second

X4: Day

X5: Month

X6: Year

X7: Milisecond

Example:

Query: TIME?

Response: TIME?,4,35,43,1,1,2000 (04:35:43 01/01/2000)

Note: All time operations are UTC based.

## SET CLOCK

Function: This command sets the RTC (real-time clock) date and time information of the Satrak ACU.

Request: TIME=,X1,X2,X3,X4,X5,X6

Parameters:

X1: Hour

X2: Minute

X3: Second

X4: Day

X5: Month

X6: Year

Example:

Request: TIME=,16,43,0,17,3,2017 (16:43:00 17/03/2017)

Note: All time operations are UTC based.

## QUERY START/STOP TIME SETTINGS

Function: This command reads the tracking start and stop time settings of the SATRAK system.

Query: START_STOP_TIME?

Response: START_STOP_TIME?,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11,X12

Parameters:

X1: Start hour

X2: Start minute

X3: Start second

X4: Start day

X5: Start month

X6: Start year

X7: Stop hour

X8: Stop minute

X9: Stop second

X10: Stop day

X11: Stop month

X12: Stop year

Example:

Query: START_STOP_TIME?

Response: START_STOP_TIME?,19,37,30,23,6,2017,19,39,0,23,6,2017

(Start Time: 19:37:30 23/06/2017, Stop Time: 19:39:00 23/06/2017)

Note: All time operations are UTC based.

## SET START/STOP TIME SETTINGS

Function: This command sets the tracking start and stop time settings of the SATRAK system.

Request: START_STOP_TIME=,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11,X12

Parameters:

X1: Start hour

X2: Start minute

X3: Start second

X4: Start day

X5: Start month

X6: Start year

X7: Stop hour

X8: Stop minute

X9: Stop second

X10: Stop day

X11: Stop month

X12: Stop year

Example:

Request: START_STOP_TIME=,20,30,0,11,8,2017,20,35,0,11,8,2017

(Start Time: 20:30:00 11/08/2017, Stop Time: 20:35:00 11/08/2017)

Note: All time operations are UTC based.

## QUERY GENERAL SETTINGS

Function: This command reads the general settings of the SATRAK system.

Query: SETTINGS?

Response: SETTINGS?,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11,X12

Parameters:

X1: RTC auto update from GPS (1:On, 0:Off)

X2: Tracking time offset (Miliseconds)

X3: Tracking Az position offset (Degrees)

X4: Tracking El position offset (Degrees)

X5: Go to start before (Miliseconds)

X6: Position Base (0:Resolver+Encoder, 1: Resolver)

X7: Minimum Elevation Angle

X8: North Offset Angle

X9: SSPA Manuel Enable (0: Disable, 1:Enable, 2:Auto)

X10: SSPA Auto Enable Options (0:Only Tracking, 1: Only Carrier Lock, 2: Tracking And Carrier Lock, 3: Tracking Or Carrier Lock )

X11: TLE data reading mode (0: Read From File, 1: Read From Web, 2: Read From FTP)

X12: Elevation Trashold Angle

Example:

Query: SETTINGS?

Response: SETTINGS?,1,100,1.0,2.0,60000,1,3.0,6.0,1,1,1,2.0

## SET GENERAL SETTINGS

Function: This command sets the general settings of the SATRAK system.

Note: A parameter data can be ‘?’, that means no changes are made for that data in this system.

Request: SETTINGS=,X1,X2,X3,X4,X5,X6,X7,X8,X9,X10,X11

Parameters:

X1: RTC auto update from GPS (1:On, 0:Off)

X2: Tracking time offset (Miliseconds)

X3: Tracking Az position offset (Degrees)

X4: Tracking El position offset (Degrees)

X5: Go to start before (Miliseconds)

X6: Position Base (0:Resolver+Encoder, 1: Resolver)

X7: Minimum Elevation Angle

X8: North Offset Angle

X9: SSPA Manuel Enable (0: Disable, 1:Enable, 2:Auto)

X10: SSPA Auto Enable Options (0:Only Tracking, 1: Only Carrier Lock, 2: Tracking And Carrier Lock, 3: Tracking Or Carrier Lock )

X11: Elevation Trashold Angle

Example:

Request: SETTINGS=,1,100,1.0,2.0,60000,1,?,6.0,0,2,3.0

## QUERY ANGLE SETTINGS

Function: This command reads the angle settings of the SATRAK system.

Query: ANGLE_SETTINGS?

Response: ANGLE_SETTINGS?,X1,X2,X3,X4

Parameters:

X1: X sensor reference

X2: X angle reference

X3: Y sensor reference

X4: Y angle reference

Example:

Query: ANGLE_SETTINGS?

Response: ANGLE_SETTINGS?,39049,160.0,36838,40.0

## SET ANGLE SETTINGS

Function: This command sets the angle settings of the SATRAK system.

Request: ANGLE_SETTINGS=,X1,X2,X3,X4,X5

Parameters:

X1: Auto/manual

(1: Auto; current mechanic positions are saved as sensor references, X2 and X4 ignored.)

(0: Manual; X2 and X4 are saved as sensor references.)

X2: X sensor reference

X3: X angle reference

X4: Y sensor reference

X5: Y angle reference

Example1:

Request: ANGLE_SETTINGS=0,39049,160.0,36838,40.0

Example2:

Request: ANGLE_SETTINGS=1,0,160.0,0,40.0

## QUERY GPS INFO

Function: This command reads the gps position information from the GPS of SATRAK.

Query: GPS_DATA?

Response: GPS_DATA?,X1,X2,X3,X4,X5,X6,X7,X8,X9

Parameters:

X1: Latitude

X2: Longitude

X3: GPS Data Valid

X4: Hour

X5: Minute

X6: Second

X7: Day

X8: Month

X9: Year

Example:

Query: GPS_DATA?

Response: GPS_DATA?,41.01N,29.17E,1,6,0,2,19,10,17

## QUERY TIME-STAMPED POSITION

Function: This command reads the X and Y positions with time-stamped information.

Query: TPOS?

Response: TPOS?,X1,X2,X3,X4,X5,X6,X7,X8,X9

Parameters:

X1: X angle

X2: Y angle

X3: Time hour

X4: Time minute

X5: Time second

X6: Time day

X7: Time month

X8: Time year

X9: Time milisecond

Example:

Request : TPOS?

Response: TPOS?,306.00,48.74,15,33,41,17,10,2017,818

## TRACKING START/STOP

Function: This command starts or stops the tracking operation.

Request: TRACK=,X1

Parameters:

X1: Start/stop (1: Start, 0: Stop)

Example: Start the tracking operation

Request: TRACK=,1

Example: Stop the tracking operation

Request: TRACK=,0

## AUTO TRACKING ON/OFF

Function: This command starts or stops the tracking operation.

Request: TRACK_AUTO_ENABLE=,X1

Parameters:

X1: On/Off (1: On, 0: Off)

Example: Enable auto tracking

Request: TRACK_AUTO_ENABLE =,1

Example: Disable auto tracking

Request: TRACK_AUTO_ENABLE =,0

## CLOCK UPDATE BY GPS

Function: This updates the Satrak ACU system clock if the gps is locked. In Satrak system, if auto clock update is enabled then the system clock is updated automatically when the gps is locked.

Request: RTC_REUPDATE=

Parameters:

No parameters

Example:

Request: RTC_REUPDATE=

## QUERY GPS SETTINGS

Function: This command reads the gps settings of the SATRAK system.

Query: GPS_SETTINGS?

Response: GPS_SETTINGS?,X1,X2,X3

Parameters:

X1: Auto/Manual

(1: Auto; location is used read from GPS module.)

(0: Manual; location is used read from settings. (X2 and X3))

X2: Manual location latitude

X3: Manual location longitude

Example:

Query: GPS_SETTINGS?

## SET GPS SETTINGS

Function: This command sets the gps settings of the SATRAK system.

Query: GPS_SETTINGS=,X1,X2,X3

Parameters:

X1: Auto/Manual

(1: Auto; location is used read from GPS module.)

(0: Manual; location is used read from settings. (X2 and X3))

X2: Manual location latitude

X3: Manual location longitude

Example:

Request: GPS_SETTINGS=,0,41.02N,29.17E

## QUERY SATELLITE SETTINGS

Function: This command reads the satellite settings of the SATRAK system.

Query: SATELLITE?

Response: SATELLITE?,X1,X2,X3,X4,X5

Parameters:

X1: TLE File Name

X2: Satellite Name

X3: TLE File Update Period

X4: Norad Id

X5: Maximum Elevation

Example:

Query: SATELLITE?

## SET SATELLITE SETTINGS

Function: This command sets the satellite settings of the SATRAK system.

Query: SATELLITE=,X1,X2,X3,X4

Parameters:

X1: TLE File Name

X2: Satellite Name

X3: TLE File Update Period

X4: The choice to continue the current tracking (0: Stop Tracking, 1:Continue Current Tracking)

Example:

Request: SATELLITE=,iridium.txt,IRIDIUM 70 [+],1,1

## QUERY TLE HTTP SETTINGS

Function: This command reads the reading TLE settings from Web.

Query: SATELLITE_HTTP?

Response: SATELLITE_HTTP?,X1,X2,X3

Parameters:

X1: Web Address

X2: TLE File Update Period (days)

X3: Update Error Action (0: Stop Tracking, 1:Continue From Last Tracking Data)

Example:

Query: SATELLITE_HTTP?

## SET TLE READING VIA HTTP SETTINGS

Function: This command sets the satellite reading settings from web of the SATRAK system.

Query: SATELLITE_HTTP=,X1,X2,X3,X4

Parameters:

X1: Web Address

X2: TLE File Update Period (days)

X3: Update Error Action(0: Stop Tracking, 1:Continue From Last Tracking Data)

X4: The choice to continue the current tracking (0: Stop Tracking, 1:Continue Current Tracking)

Example:

Request: SATELLITE_HTTP=, https://62bb0cca7bdbe01d529645e6.mockapi.io/Satellite,1,1,0

## QUERY TLE FTP SETTINGS

Function: This command reads the reading TLE settings from Ftp.

Query: SATELLITE_FTP?

Response: SATELLITE_FTP?,X1,X2,X3,X4,X5,X6,X7,X8

Parameters:

X1: IP Address

X2: User Name

X3: Password

X4: Port

X5: TLE File Name

X6: Satellite Name

X7: TLE File Update Period (days)

X8: Update Error Action(0: Stop Tracking, 1:Continue From Last Tracking Data)

Example:

Query: SATELLITE_FTP?

## SET TLE READING VIA FTP SETTINGS

Function: This command sets the satellite reading settings from ftp of the SATRAK system.

Query: SATELLITE_FTP=,X1,X2,X3,X4,X5,X6,X7,X8

Parameters:

X1: IP Address

X2: User Name

X3: Password

X4: Port

X5: TLE File Name

X6: Satellite Name

X7: TLE File Update Period (days)

X8: Update Error Action(0: Stop Tracking, 1:Continue From Last Tracking Data)

Example:

Request: SATELLITE_FTP=, 10.0.1.2,username,password,22,1,1

## QUERY SYSTEM IMAGE FILE

Function: This command reads the loading status of image file.

Query: LOAD_IMAGE?

Response: LOAD_IMAGE?,X1

Parameters:

X1: Update Status

Example:

Query: LOAD_IMAGE?

## SET IMAGE SETTINGS

Function: This command sets the satellite settings of the SATRAK system.

Query: LOAD_IMAGE =,X1

Parameters:

X1: Image File Name

Example:

Request: SATELLITE=,sysImage.json

## QUERY REDUCTION

Function: This command reads the reduction values.

Query: REDUCTION?

Response: REDUCTION?,X1,X2

Parameters:

X1: x-axis reduction

X2: y-axis reduction

Example:

Query: REDUCTION?

## SET REDUCTION

Function: This command sets the reduction values of the SATRAK system.

Query: REDUCTION=,X1,X2

Parameters:

X1: x-axis reduction

X2: y-axis reduction

Example:

Request: REDUCTION=,4000,4000

# TLE FILE UPLOAD

## TLE FILE UPLOAD OPERATION

TLE file is uploaded to the controller with SFTP protocol. Username and password are given below.

Username: sftpsatr

Password: satrak

TLE file should be uploaded to the “/svs/satrak/tle” folder in the Satrak controller file system.